TASK 3

Degrees of Freedom:

Degree of freedom of a body is the total number of variables which can effect its position

and orientation in a system (with its given constraints ).

Links:

Rigid elements of a mechanism.

Types of Links:

a) Rigid Link:

do not experience any deformation while in motion

b) Flexible Link

can experience temporary deformation while in motion

c) Fluid Link

when fluid pressure is used to transmit motion

Joints:

Two links in contact which produce a desired constrained/predictable motion is called a joint.

Classification based on relative motion:

a) Sliding pair:

When the relative motion between the two bodies is a sliding motion

along one axis.

Therefore has only one DOF -  along the axis of sliding.

b) Turning pair:

When the relative motion is turning of the link(s) about one axis.

DOF - 1

c)Rolling pair:

Relative motion is rolling

the DOF is considered 1 if slipping doesn't occur in the system, as then

the translation and rotational motion happen in a fixed ration so knowing

only one of them will be enough to determine the motion.

The dof is 2 if slipping can occur in the system, as then it isn't necessary

for rotational and translational motion to happen in a fixed ratio.

d) Screw pair:

when the relative motion is the translational motion of one link along one

axis and rotation about the same, causing the link to take a helical path.

Since the pitch(distance between two successive thread peaks) of a

screw is fixed, the ratio of distance travelled to rotations

completed is always the same, hence they are not two independent

motions.

So the DOF of a screw pair is only 1.

e) Spherical pair:

when a ball element moves inside a cup element.

since the ball can rotate about all three axes, the DOF is 3.

Classification based on nature of contact:

a) lower pairs:

when the contact between the two links is a common surface

b) higher pairs:

when the contact between tow links is a common point or line.

Mechanisms:

The combination of links and joints such that they produce the desired motion -

predictable motion.

Planar Mechanism:

A kind of mechanism where all the links move along the same or parallel planes,

constricting motion to 2D.

Grubler's formula:

used to determine the DOF of a planar mechanism - the minimum number of

independent values needed to predict the motion of the mechanism.

Formula:

F= 3(L-1) - 2J1- J2

here L is the total number of links, and the coefficient 3 being multiplied gives us

the maximum possible degrees of freedom or total unconstrained freedom,

as each link in a plane has 3 degrees of freedoms( translational about x and y

and rotation about any axis perpendicular to the plane).

The 1 being subtracted from L is due to the stipulation that at least one link has

to be fixed to the ground.

As the addition of joints removes degrees of freedom, the next two terms account

for joints.

J1 accounts for lower pairs, as they only allow one DOF, the coefficient of J1 is 2.

J2 accounts for higher pairs, as they only allow 2 DOF, the coefficient of J2 is 1.